Assignment No .1

Ques1: Explain the fundamental limitation of a communication systam?

Ques2: What is the difference between analog and digital communication      system?

Ques3: What is modulation? What is its basic purpose in Electronic                 communication? Discuss the various needs and benefits of modulation?

Ques4: What is the difference between low-level amplitude modulation and    high-level amplitude modulation?

Ques5: Draw waveform for AM signal for undermodulation, overmodulation and 100% modulation?

Ques6: What are the advantages, disadvantages and application of AM?

Ques7: Describe balance Modulator using FET with the help of suitable diagram?

Ques8: Differentiate between envelope detector and synchronous detector?

Assignment No .2

Ques1: What is Hilbert transform and inverse Hilbert transform? What are             the applications and advantages of Hilbert transform?

Ques2: Explain Inphase and Quadrature representation of SSB transmission?

Ques3: Why SSB transmission is not used for broadcasting?

Ques4: Explain vestigial side band modulation? What is its bandwidth?

Ques 5: Which modulation is used for transmission of picture and speech in TV transmission and why?

Ques 6: Comapare AM, DSB-SC and SSB-SC modulation?

Ques7: What is Quadrature null effect? How to overcome its effect?

Assignment No. 3

Ques 1: What is low noise amplifier and noise limiter?

Ques 2: What are the salient features of broadcast radio receivers?

Ques3: What are the main functions served by an IF amplifier and what factors govern the choice of intermediate frequency?

Ques4: What are the charactertics of RF amplifier?

Ques5: Distinguish between simple AGC and Delayed AGC?

Ques6: What is tracking error.Also draw a typical tracking error curve.

Ques 7: What is meant by the term adjacent channel selectivity?

Ques 8: For a broadcasting superhetrodyne AM receiver having no RF amplifier, the loaded Quality Factor Q of the antenna coupling circuit is 100. Now if the intermediate frequency is 455 kHz, then determine the image frequency and its rejection ratio at an incoming frequency of 100 KHz.

Assignment No.4

Ques1:An antenna transmits an AM signal having total power contents of 15KHz .Determine the power being transmitted at the carrier frequency and each of the sideband when the percent modulation if 85%?

Ques2:  A bandwidth of 12 MHz becomes available for assignment. If assigned for TV broadcast service, only two channels could be accommodated. Determine the number of AM stations that could broadcast simultaneously if the maximum modulated frequency is limited to 5 KHz.

Ques3: A 90 kHz bandwidth is to accommodate six AM broadcasts simultaneously. What maximum modulating frequency must each station be limited to?

Ques4: The following AM signal

S(t)=Ac[1+kam(t)]cos (2πfct)

Is applied to the system in the fif a.Assuming that |kam(t)|<1 for all t and message signal m(t) is limited to the interval –fm≤ f≤fm and that the carrier frequency fc > 2fm, show that m(t) can be obtained from the squarer –rooter output v3(t)

Low-Pass Filter

Square Router

Squarer

x(t)

v1(t)

v2(t)

v3(t)

v1(t)=s2(t)

v3(t)=√ v2(t)

Ques5: A SSB transmission contains 15 KW.This transmission is to be replaced by a standard amplitude modulated signal with the same power content. Determine the power content of the carrier and each of the sidebands when the percent modulation is 60%.

Ques6: A tuned radio frequency receiver (TRF) has to be designed with a single tuned circuit using 9×10-6 Henry inductor.The ideal 10 KHz bandwidth occurs at 1100 KHz.

- Determine the capacitance range of the variable capacitor in the LC tank circuit required to tune the receiver in 550 to 1550 ranges.

- Determine the bandwidth of the receiver at 550 and 1550 Kz.

Assignment 5

Ques1: Explain the difference between Narrowband and Wideband FM?

Ques2: Write a short note on Bessel function?

Ques3: Define the following terms for Fm wave;

- Carrier Swing

- Frequency Deviation

- Per cent Moidulatio

- Instantaneous frequency

Ques 4: What is Carson’s rule?

Ques5: Compare AM, FM, PM.

Ques6: Explain PLL-FM demodulator?what is its advantages over other FM demodulators?

Ques7: Given an angle-modulated signal

Xc(t)= 10cos[(108)πt + 5sin2π(103)t]

Determine the maximum phase deviation and the maximum frequency deviation.

Ques8: Determine the modultion index of an FM signal which is being broadcast in the 88-108 MHz band.This FM wave has a carrier swing of 130KHz.

Assignment 6

Ques1: What is noise? What is its effect in electronic signal?

Ques2: Explain how noise figure of cascaded stages can be calculated?

Ques3 : Noise figure of individual two stage amplifier is 2dB with power gain of 12 dB and 6 dB with power gain of 10 dB resp. Find the overall noise figure in dB?

Ques4: What is white noise? Write the expression for PSD of white noise.

Ques5: Two resistors R1 and R2 at absolute T1 and T2 are connected in series to form a wite noise source. Find the equivalent noise temperature Teq.

Ques6: Determine the noise figure of an amplifier for which noise equivalent resistance Req= 2600 and Rt= 500 ohm. It is driven by a generator whose output impedance is 50 ohm. It is given that this constitutes a large enough mismatch.