ASSIGNMENT 1

(Amplitude Modulation )

Ques1:AM wave has total power content of 600 watts. Find the percentage modulation of the         signal if each of the side bands contains 75 watts.

Ques2.A certain AM Transmitter radiates 9*10³ Watt with the carrier unmodulated and 10.125*10³ W when carrier is sinusoidally modulated. Find

(a) modulation index(m)

(b) Percent of modulation.

Ques3: Determine the carrier power if an AM broadcast radio transfer 25kw of power

with 60%  modulation.

Ques4: What are the advantages of Suppressed Carrier modulation.

Ques3. Explain the need of modulation.

Ques4.Compare various AM Systems like AM, DSB-SC, SSB and VSB System.

Ques5. Find the percentage power saving when the carrier wave and one of the side bands are suppressed in the AM wave modulated to a depth of 50%?

Ques6. An AM wave processes total power content of 600w. Find % modulation of signal if each of SB contains 75w.

Ques7: The ac rms antenna current of an AM transmitter is 6.2A when unmodulated and rises to 6.7 A when modulated. Calculate the percentage modulation.

Ques8 : Suppose that the modulating signal is 𝑚(𝑡) = 2 cos(2𝜋𝑓𝑚𝑡) and the carrier signal is 𝑥𝐶(𝑡) = 𝐴𝐶 cos(2𝜋𝑓𝑐𝑡).

Which one of the following is a conventional AM signal without over-modulation?