ASSIGNMENT 2

( Sampling & Angle Modulation)

Ques1: State and explain Sampling theorem of bandpass signal?

Ques2: What is aliasing and how to overcome aliasing?

Ques3: Compare WBFM & NBFM and define Carson s rule for both.

Ques4:  What is Apertue Effect and how can it be removed?

Ques5: Differentiate pulse amplitude modulation, Pulse width modulation and Pulse position modulation.

Ques6: Determine BW of NBFM which is generated by a 4 KHz audio signal modulating a 125 MHz carrier.

Ques7: Determine the Nyquist rate and Nyquist interval for following signals

- Sin2(100πt)

- Sin(100πt)sin(1000πt)

- Sinc^2(100t)

Ques8: Given an angle-modulated signal

Xc(t)= 10cos[(108)πt + 5sin2π(103)t]

Determine the maximum phase deviation and the maximum frequency deviation.

Ques9: Determine the modultion index of an FM signal which is being broadcast in the 88-108 MHz band.This FM wave has a carrier swing of 130KHz.

Ques10: Calculate the Nyquist rate and Nyquist Interval for the signal

S(t)= 2π cos(5000πt) cos(500πt )

Ques11: In case of FM if the message frequency is 15 KHz and the frequency deviation is 75 KHz, then calculate the bandwidth requirement for transmission.