Question Bank

- Define Signal.

- Define system.

- What are the major classifications of the signal?

- Define discrete time signals and classify them.

- Define continuous time signals and classify them.

- Define discrete time unit step &unit impulse.

- Define continuous time unit step and unit impulse.

- Define unit ramp signal.

- Define periodic signal and non-periodic signal.

- Define even and odd signal ?

- Define Energy and power signal.

- Define unit impulse function.

- Define continuous time complex exponential signal.

- What is continuous time real exponential signal.

- What is continuous time growing exponential signal?

- State the BIBO criterion for stability.

- Find whether the signal given by x (n) = 5cos (6 πn) is periodic .

- For the systems represented by the following functions.

- Determine whether   every system is

  - stable (2) Causal (3) linear (4) Shift invariant

    - T[x(n)]= ex(n) (ii) T[x(n)]=ax(n)+6

- Determine whether the following systems are static or Dynamic, Linear or Non-linear, Shift variant or Invarient, Causal or Non-causal, Stable or unstable.

  - y(t) = x(t+10) + x2(t) (ii) dy(t)/dt + 10 y(t) = x(t)

- Explain about the properties of continuous time fourier series.

- Find the fourier coefficients of the given signal.

- x(t) = 1+ sin 2ωt + 2 cos 2ωt + cos (3ωt + π/3)

- Determine the Fourier series coefficient of exponential representation of x(t)

- x(t) =  1, ׀t׀ <T1

- 0, T1< ׀t׀ < T/ 2

- Find which of the following signal are energy or power signals.

    - x(t)=e-3t u(t)  b) x(t) = ej(2t+π/4)

- Define continuous time system.

- Define Fourier transform pair.

- Write short notes on dirichlets conditions for fourier transform.

- Explain how aperiodic signals can be represented by fourier transform.

- State convolution property in relation to fourier transform.

- State parseval’s relation for continuous time fourier transform.

- What is the use of Laplace transform?

- What are the types of laplace transform?

- Define Bilateral and unilateral laplace transform.

- Define inverse laplace transform.

- State the linearity property for laplace transform.

- State the time shifting property for laplace transform.

- Region of convergence of the laplace transform.

- What is pole zero plot.

- State initial value theorem and final value theorem for laplace transform.

- State Convolution property.

- Define a causal system.

- What is meant by linear system?

- Define time invariant system.

- Define stable system?

- Define memory and memoryless system.

- Define invertible system.

- What is superposition property?

- Find the fourier transform of x(t)=cos(Ωt)

- Determine the inverse laplace of the following functions.

      - 1/s(s+1)  2) (3s2 +8s+6)/(s+2)(s2+2s+1)

- Explain about the classifications of continuous time system.

- A system is described by the differential equation.     d2y(t)/dt2+3dy(t)/dt+2y(t)=dx(t)/dt if y(0) =2;dy(0)/dt = 1 and x(t)=e-t u(t)

- Use laplace transform to determine the response of the system to a unit step input applied at t=0.

- Obtain the transfer function of the system when y(t) = e-t-2 e-2t+ e-3t and       x(t)= e-0.5t

57)Discuss the condition on stability of an LTI system based on Laplace domain representation.

58)Bring the equivalence between Laplace transform and Fourier transform.

- Explain the properties of laplace transform

- Find the impulse and step response of the following systems

H(s) =  10/(s2+6s+10 )

- For the transfer function

H(s) = (s+10)/( s2+3s+2)

find the response due to input x(t) = sin2(t) u(t)

- Find the fourier transform of triangular pulse

- x(t) = Δ(t/m) ={1-2|t|/m      |t|<m

      - 0 otherwise

- The input and output of a causal LTI system are related by the differential equation.

d2y(t)/dt2+6dy(t)/dt+8y(t)=2x(t)

Find the impulse response of the system.

What is the response of this system if x(t) = t e-2t u(t)

- Consider a causal LTI system with frequency response.

H(jΩ) = 1/ (jΩ +2)

For a particular input x(t) this system is y(t)= e-2t u(t) - e-3t u(t)

- Why CT signals are represented by samples.

- What is meant by sampling.

- State Sampling theorem.

- What is meant by aliasing.

- What are the effects aliasing.

- How the aliasing process is eliminated.

- Define Nyquist rate. and Nyquist interval.

- Define sampling of band pass signals.

- Define Z transform.

- What are the two types of Z transform?

- Define unilateral Z transform.

- What is region of Convergence.

- What are the Properties of ROC.

- What is the time shifting property of Z transform.

- What is the differentiation property in Z domain.

- State convolution property of Z transform.

- State the methods to find inverse Z transform.

- State multiplication property in relation to Z transform.

- State parseval’s relation for Z transform.

- What is the relationship between Z transform and fourier transform.

- What is meant by step response of the DT system.

- Define unilateral Z transform.

- What is region of Convergence.

- What are the Properties of ROC.

- What is the time shifting property of Z transform.

- What is the differentiation property in Z domain.

- State convolution property of Z transform.

- State the methods to find inverse Z transform.

- State multiplication property in relation to Z transform.

- State parseval’s relation for Z transform.

- What is the relationship between Z transform and fourier transform.

- What is meant by step response of the DT system.

- Give the properties of convolution

- Determine the step response of the difference equation,

- y(n)-(1/9)y(n-2)=x(n-1) with y(-1)=1 and y(-2)=0

- Find the impulse response and step response.

y(n)-3/4y(n-1) +1/8 y(n-2) = x(n)

- Find the output y(n) of a linear time invariant discrete time system specified by the equation.

y(n)-3/2y(n-1) +1/2 y(n-2) = 2x(n) +3/2 x(n-1) when initial conditions are y(-1) =0,y(-2) = 1 and input x(n)=(1/4)n u(n)

- Determine the Nyquist sampling rate and Nyquist sampling intervals for sinc(200πt) + 3sinc2(120πt)

- Find the frequency response of the following causal system.

y(n)=1/2x(n)+x(n-1)+1/2 x(n-2)

- What are the major classifications of the signal?

- Define energy and power signals.

- Define continuous time complex exponential signal. What is continuous time real exponential signal? What is continuous time growing exponential signal?

- State the BIBO criterion for stability.

- Find whether the signal given by x (n) = 5cos (6n) is periodic.

- State the convolution Integral for CT LTI systems?

- Define convolution sum with its equation

- What is meant by impulse response of any system?

- Given the impulse response h(t), state the conditions for stability and causality.

- Determine the convolution of the signals X(n)={2,-1,3,2} & h(n)={1,-1,1,1}.

- Define Natural, Forced and complete response?

- Write down the exponential and trigonometric form of the Fourier series representation of a periodic signal? Write short notes on Dirichlets conditions.

- State Parseval’s theorem for continuous time periodic signals.

- Define Fourier transform pair. Write short notes on dirichlets conditions for fourier transform.

- What is the use of Laplace transform? Define Bilateral and unilateral laplace transform.

- What are the properties of Region of convergence of Laplace transform.

- Define invertible system.

- What is the Laplace transform of the function x(t)=u(t)-u(t-2)?

- What is ROC in Z transforms?

- What is the relation between DTFT and Z transform?

- Define system function.

- What is meant by sampling? What are the effects aliasing? Define Nyquist rate and Nyquist interval.

- Determine the Nyquist sampling rate and Nyquist sampling intervals for sinc(200πt) + 3sinc2 (120πt)

- What is natural response?

- What is zero input Response?

- What is forced response?

- What is complete response